Algorithm:

Initialization Phase (High Priority):

- Start Serial Communication:

  - Begin serial communication at 9600 baud to allow for monitoring and debugging.

- Allocate Memory for Sensors and Actuators:

  - Dynamically allocate memory for three UltrasonicSensor and three Actuator objects.

  - If memory allocation fails, print an error message and halt the program.

- Initialize Sensor and Actuator Properties:

  - Assign triggerPin and echoPin to each UltrasonicSensor object.

  - Assign vibratorPin and frequency to each Actuator object.

- Set Pin Modes:

  - Set ultrasonic sensors' triggerPin as OUTPUT and echoPin as INPUT.

  - Set each vibratorPin and BUZZER_PIN as OUTPUT.

  - Set LIGHT_SENSOR_PIN as INPUT.

Main Loop Phase (Prioritized Tasks):

- Priority 1: Obstacle Detection (Most Critical)

  - For each UltrasonicSensor:

    - Call measureDistance(&sensors[i]) to measure the distance.

    - Print the measured distance to the Serial monitor.

    - If the distance is less than or equal to MAX_DISTANCE:

      - Activate the corresponding actuator (digitalWrite(vibrators[i].vibratorPin, HIGH)).

      - Play a tone on the BUZZER_PIN at the actuator's frequency for 200 milliseconds.

    - If the distance is greater than MAX_DISTANCE:

      - Deactivate the actuator (digitalWrite(vibrators[i].vibratorPin, LOW)).

- Priority 2: Temperature Monitoring (Moderate Priority)

  - Read the analog value from TEMPERATURE_PIN.

  - Convert the value to temperature in Celsius.

  - Print the temperature to the Serial monitor.

  - If a switch connected to pin 6 is HIGH:

    - Check if the temperature is outside the safe range (e.g., < 25°C or > 35°C).

    - If the temperature is out of range, sound the buzzer at 1000 Hz for 300 milliseconds.

- Priority 3: Light Level Monitoring (Lower Priority)

  - Read the analog value from LIGHT_SENSOR_PIN.

  - Print the light level to the Serial monitor.

  - If a switch connected to pin 5 is HIGH:

    - Check if the light level is below 200.

    - If the light level is below the threshold, sound the buzzer at 1000 Hz for 500 milliseconds.

Repeat Cycle:

- Include a delay of 300 milliseconds at the end of each loop iteration to avoid rapid sensor polling and ensure stability.

Cleanup Phase (Memory Management):

- Call cleanup() to free the dynamically allocated memory when needed, such as during shutdown or error handling.

Priority Breakdown:

- Obstacle Detection is the highest priority because it directly triggers the actuators and buzzer to alert the user of immediate obstacles.

- Temperature Monitoring comes next, as sudden temperature changes could indicate potential safety hazards.

- Light Level Monitoring is the least critical and is checked last because its alerts may not be as time-sensitive as the others.

High-Level Flow:

- Setup Phase → Initialize the system and allocate resources.

- Main Loop:

  - Step 1: Measure distance and handle obstacles.

  - Step 2: Check and report temperature.

  - Step 3: Check and report light level.

- Delay → Wait for 300 milliseconds.

- Repeat the loop until the program is manually stopped or halted